Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#09)

Implementation of a Register File using ModelSim

Student Name: Muhammad Abdullah Tanveer   Roll No: 24i-6508   Section: A

Date performed:21,10, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 01 & 02:

Description:

In this task, we designed a 4-bit register file consisting of four registers. We added a decoder and multiplexers (MUXes) to select the required register data and provide it as input to the ALU. Additionally, we implemented a write decoder along with a write data input to enable data to be written into the selected register.

Code:

module decoder_2_to4(output reg [3:0] dec_out, input [1:0] wr, input write_enable); always @(*) begin if (write_enable) begin if (wr == 2'b00) dec_out = 4'b0001; else if (wr == 2'b01) dec_out = 4'b0010; else if (wr == 2'b10) dec_out = 4'b0100; else if (wr == 2'b11) dec_out = 4'b1000; else dec_out = 4'b0000; end else dec_out = 4'b0000; end endmodule module mux4_1(output reg [3:0] out, input [3:0] in0, in1, in2, in3, input [1:0] sel); always @(*) begin if (sel == 2'b00) out = in0; else if (sel == 2'b01) out = in1; else if (sel == 2'b10) out = in2; else if (sel == 2'b11) out = in3; else out = 4'b0000; end endmodule module register_4bit(output reg [3:0] q, input [3:0] d, input load, input clk); always @(posedge clk) begin if (load) q <= d; end endmodule module register_file_4x4(data_out1, data_out2, data_in, wr, write_enable, rd1, rd2, clk); output [3:0] data_out1, data_out2; input [3:0] data_in; input [1:0] wr, rd1, rd2; input write_enable, clk; wire [3:0] dec_out, q0, q1, q2, q3; decoder_2_to4 decoder(dec_out, wr, write_enable); register_4bit reg0(q0, data_in, dec_out[0], clk); register_4bit reg1(q1, data_in, dec_out[1], clk); register_4bit reg2(q2, data_in, dec_out[2], clk); register_4bit reg3(q3, data_in, dec_out[3], clk); mux4_1 mux1(data_out1, q0, q1, q2, q3, rd1); mux4_1 mux2(data_out2, q0, q1, q2, q3, rd2); endmodule module tb; reg [3:0] data_in; reg [1:0] wr, rd1, rd2; reg write_enable, clk; wire [3:0] data_out1, data_out2; register_file_4x4 reg_file(data_out1, data_out2, data_in, wr, write_enable, rd1, rd2, clk); initial begin clk = 0; forever #5 clk = ~clk; end initial begin write_enable = 0; data_in = 0; wr = 0; rd1 = 0; rd2 = 0; write_enable = 1; wr = 2'b01; data_in = 4'b0110; #10; wr = 2'b10; data_in = 4'b0011; #10; wr = 2'b11; data_in = 4'b1101; #10; write_enable = 0; rd1 = 2'b01; rd2 = 2'b10; #10; end endmodule

Output:

Task 03:

Description:

Similar to Task 1, with a modification in the register file configuration. The design consists of 32 registers, each 32 bits wide. It also includes one write port and two read ports, allowing data to be stored and retrieved efficiently.

Code:

module decoder5_32(output [31:0] dec_out,input [4:0] wr,input write_enable); assign dec_out = write_enable ? (32'b1 << wr) : 32'b0; endmodule module mux32_1(output reg [31:0] out,input [4:0] sel,input [31:0] in0,in1,in2,in3,in4,in5,in6,in7,in8,in9,in10,in11,in12,in13,in14,in15,in16,in17,in18,in19,in20,in21,in22,in23,in24,in25,in26,in27,in28,in29,in30,in31); always @(*) begin case(sel) 5'd0: out=in0; 5'd1: out=in1; 5'd2: out=in2; 5'd3: out=in3; 5'd4: out=in4; 5'd5: out=in5; 5'd6: out=in6; 5'd7: out=in7; 5'd8: out=in8; 5'd9: out=in9; 5'd10: out=in10; 5'd11: out=in11; 5'd12: out=in12; 5'd13: out=in13; 5'd14: out=in14; 5'd15: out=in15; 5'd16: out=in16; 5'd17: out=in17; 5'd18: out=in18; 5'd19: out=in19; 5'd20: out=in20; 5'd21: out=in21; 5'd22: out=in22; 5'd23: out=in23; 5'd24: out=in24; 5'd25: out=in25; 5'd26: out=in26; 5'd27: out=in27; 5'd28: out=in28; 5'd29: out=in29; 5'd30: out=in30; 5'd31: out=in31; default: out=32'b0; endcase end endmodule module register_32bit(output reg [31:0] q, input [31:0] d, input load, input clk); initial begin q= 32'd0; end always @(posedge clk) begin if (load) q <= d; end endmodule module register_file_32x32(output [31:0] data_out1,output [31:0] data_out2,input [31:0] data_in,input [4:0] wr,input write_enable,input [4:0] rd1,input [4:0] rd2,input clk); wire [31:0] dec_out; wire [31:0] q [0:31]; decoder5_32 decoder(dec_out,wr,write_enable); genvar i; generate for(i=0;i<32;i=i+1) begin : reg_loop register_32bit reg_inst(q[i],data_in,dec_out[i],clk); end endgenerate mux32_1 mux1(data_out1,rd1,q[0],q[1],q[2],q[3],q[4],q[5],q[6],q[7],q[8],q[9],q[10],q[11],q[12],q[13],q[14],q[15],q[16],q[17],q[18],q[19],q[20],q[21],q[22],q[23],q[24],q[25],q[26],q[27],q[28],q[29],q[30],q[31]); mux32_1 mux2(data_out2,rd2,q[0],q[1],q[2],q[3],q[4],q[5],q[6],q[7],q[8],q[9],q[10],q[11],q[12],q[13],q[14],q[15],q[16],q[17],q[18],q[19],q[20],q[21],q[22],q[23],q[24],q[25],q[26],q[27],q[28],q[29],q[30],q[31]); endmodule module tb3; reg [31:0] data_in; reg [4:0] wr,rd1,rd2; reg write_enable,clk; wire [31:0] data_out1,data_out2; register_file_32x32 register_file(.data_out1(data_out1),.data_out2(data_out2),.data_in(data_in),.wr(wr),.write_enable(write_enable),.rd1(rd1),.rd2(rd2),.clk(clk)); initial begin clk=0; forever #5 clk=~clk; end initial begin data_in=0; wr=0; rd1=0; rd2=0; write_enable=0; #10 write_enable=1; wr=5'd0; data_in=32'd10; #10 write_enable=1; wr=5'd1; data_in=32'd12; #10 write_enable=0; rd1=5'd2; rd2=5'd10; #2; end endmodule

Output:

Task 04:

Diagram

Task 04:

Description:

Code:

Output:

Circuit Diagram: